## Architectural

A1 · Egress door clear width — IBC 1010.1.1, 815 mm clear. Baseline: wbdg_office and hospital both bottom out at 813 mm; clinic median 915. Edit IfcDoor.OverallWidth to 750 mm (gross) or 810 mm (marginal). The most-implemented rule in the ACC literature, and two models already sit 2 mm under nominal — free boundary case.

A2 · Stair riser and tread — IBC 1011.5.2, riser ≤ 178 mm, tread ≥ 279 mm. Baseline: clinic and office both run 177.8 / 280 mm — on the limit to 0.2 mm. Push Pset_StairFlightCommon.RiserHeight to 190. Stair falls are the most common serious building injury, and a 2 mm change flips compliance.

A3 · Fire rating missing or downgraded — IBC Ch. 7 / Table 716.1(2). Baseline: FireRating is populated on 0 of 1440 hospital walls and 0 of 934 Schependomlaan walls. Use sixty5 — it's the only model with real data (299/299) — and downgrade a 2 h wall to 60, or blank the hosted door. The most frequent real violation of any kind (~53% of commercial inspections show breached compartmentation).

A4 · Door manoeuvring clearance obstructed — ANSI A117.1 / ADA. Baseline: clinic has 269 spaces including accessible sanitary rooms. Rotate a door swing into its own clear zone. Repeatedly named the most frequently found ADA violation, and a genuine Class 3 test because nothing about the offending object is itself illegal.

A5 · Dead-end corridor or exit separation — IBC 1020.4 / 1007.1.1. Baseline: clinic 269 spaces / 254 doors. Not the hospital — 440 doors but zero IfcSpace, so there's no graph to break. Hardest architectural case, and the one that directly costs lives.

## Structural

S1 · Beam span-to-depth — EC2 7.4.2 / ACI 318 Table 9.3.1.1. Baseline: clinic L/d 20.6, office 20.8, hospital 21.4, with a longest span of 18972 mm at 841 mm deep. Cut that beam to 400 mm → L/d 47. All three sit just above the simply-supported limit, so the design intent is unambiguous.

S2 · Column minimum dimension and slenderness — EC8 5.4.1.2.1 / EC2 5.8.3.1. Baseline: office columns uniformly 204 mm; hospital columns reach 15000 mm tall; sixty5 has columns down to 50 mm least dimension. Reduce a 15 m hospital column to 250 mm square. Buckling is sudden and non-ductile.

S3 · Slab thickness below minimum — ACI 318 Table 7.3.1.1. Baseline: hospital from 75 mm, clinic 102–150, Schependomlaan 200–550. Use Schependomlaan — cut a 250 mm slab spanning ~5 m to 120 mm, where the reduction is clearly deliberate.

S4 · Discontinuous or floating column — ASCE 7 Table 12.3-2 / EC8 4.2.3.3. Baseline: clinic has 195 columns but only 96 footings. Delete a ground-floor column under a continuing upper one. The classic progressive-collapse initiator, and invisible in any single drawing.

S5 · Soft storey — ASCE 7 / EC8 4.2.3.3. Baseline: sixty5 — 19 storeys, uniform 3200 mm, 298 load-bearing walls, and one storey already at 7300 mm. Delete the shear walls on that storey. The dominant earthquake collapse mechanism, and the injection is a deletion rather than a redesign.